# EXTENDED DETENTION INSPECTION & MAINTENANCE CHECKLIST

# INTRODUCTION

Once construction is completed, periodic site inspections are essential in order to monitor the effectiveness and to anticipate the maintenance needs of the BMP. When conducting inspections, attention should be given not only to the BMP installed for stormwater control, but also to the conveyance system carrying runoff to the BMP and the receiving channel immediately downstream of the BMP. The conveyance channel, curbing and/or storm sewer that convey flow to the BMP or, by design, intentionally divert flows around it are all considered BMP components and must function as intended.

ED ponds shall be inspected and cleaned up annually, preferably during the Spring. The site shall be inspected at least twice a year and mowing be done every other year to manage the buffer. Watering is needed once a week during the first 2 months following installation, and then as needed during the first growing season (April-October), depending upon rainfall. If vegetation needs to be replaced, one-time spot fertilization may be needed, preferably using an organic rather than a chemical fertilizer.

Routine maintenance such as grass mowing, weeding, trash removal, mulch raking and maintenance, erosion repair, reinforcement plantings, tree and shrub pruning, and sediment removal shall be performed as necessary.

The checklists shall be signed, dated, and maintained at an accessible location with an official representative of the homeowners association, the individual or company contracted for maintenance, or the owner.

# EXTENDED DETENTION POND PRACTICES: O&M CHECKLIST

Inspection Date

Inspector

Project

Location

Date BMP Placed in Service

Date of Last Inspection

Owner/Owner’s Representative

Town Project Number

As-Built Plans available:	Y / N

Facility Type:  Level 1 ____________________________   Level 2

Ideally, Extended Detention Ponds should be inspected annually. ED Ponds are prone to a high clogging risk at the ED low-flow orifice. Ideally, the orifice should be inspected at least twice a year after initial construction. The constantly changing water levels in ED Ponds make it difficult to mow or manage vegetative growth. The bottom of ED Ponds often become soggy, and water-loving tees such as willows may invade and will need to be managed. Periodic mowing of the stormwater buffer is only required along maintenance rights-of-way and the embankment. The remaining buffer may be managed as a meadow (mowing every other year) or forest. Frequent removal of sediment from the forebay (every 5-7 years, or when 50% of the forebay capacity is filled) is essential to maintain the function and performance of the ED Pond. Sediments excavated from ED Ponds are usually not considered toxic or hazardous, so they can be safely disposed of either by land application of land filling.

Pond characteristics and functions

(check all that apply)

Water quality treatment

Channel protection

Ties into groundwater

Hydraulic Configuration:

  - On-line facility

  - Off-line facility

Type of Pre-Treatment Facility:

Sediment forebay (above ground)

Sedimentation chamber

Plunge pool

Stone diaphragm

Grass filter strip

Grass channel

Other: _____________________

| Element of BMP | Potential Problem | Problem? Y / N | Investigate? Y / N | Repaired? Y / N | How to Fix Problem | Who Will Address Problem | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Contributing Drainage Area | Adequate vegetation |  |  |  | Supplement as needed. | Owner |  |
|  | There is excessive trash and debris |  |  |  | Remove immediately. | Owner or professional |  |
|  | There is evidence of erosino and/or bare or exposed soil |  |  |  | Stabilize immediately. | Owner or professional |  |
|  | There is excessive landscape waste and yard clippings |  |  |  | Remove immediately. | Owner or professional |  |
| Pre-Treatment | There is adequate access to the pre-treatment facility |  |  |  | Establish adequate access | Professional and, perhaps, the locality |  |
|  | There is excessive trash and debris |  |  |  | Remove immediately. | Owner or professional |  |
|  | There is evidence of erosion and/or exposed soil. |  |  |  | Immediately identify and correct the cause of the erosion and stabilize the eroded or bare area. | Owner or professional |  |
| Pre-Treatment (continued) | Sediment deposits are 50% or more of forebay capacity. |  |  |  | Dredge the sediment to restore the design capacity; sediment should be dredged from forebays at least every 5-7 years, and earlier, as needed. | Professional |  |
|  | The sediment marker is not vertical. |  |  |  | Adjust the sediment depth marker to a vertical alignment | Professional |  |
|  | There is evidence of clogging |  |  |  | Clear blockages of the riser or orifice(s) and make other adjustments needed to meet the approved design specifications | Professional |  |
|  | There is dead vegetation |  |  |  | Revegetate, as needed | Owner or professional |  |

| Element of BMP | Potential Problem | Problem? Y / N | Investigate? Y / N | Repaired? Y / N | How to Fix Problem | Who Will Address Problem | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inlet | The inlet provides a stable conveyance into the pond |  |  |  | Stabilize immediately, as needed, and clear blockages. | Owner or professional |  |
|  | There is excessive trash, debris, or sediment. |  |  |  | Remove immediately | Owner or professional |  |
|  | There is evidence of erosion/undercutting at or around the inlet |  |  |  | Repair erosion damage and restabilize | Owner or professional |  |
|  | There is cracking, bulging, erosion or sloughing of the forebay dam. |  |  |  | Repair and restabilize immediately. | Professional |  |
|  | There is woody growth on the forebay dam. |  |  |  | Remove within 2 weeks of discovery. | Professional |  |
|  | There is evidence of nuisance animals. |  |  |  | Animal burrows must be backfilled and compacted. Burrowing animals should be humanely removed from the area. | Professional |  |
|  | There is more than 1 inch of settlement. |  |  |  | Add fill material and compact the soil to the design grade | Owner or Professional |  |
|  | The inlet alignment is incorrect. |  |  |  | Correct immediately. | Owner or Professional |  |
| Vegetation | Plant composition is consistent with the approved plans |  |  |  | Determine if existing plant materials are consistent with the general Wet Pond design criteria, and replace inconsistent species. | Professional |  |
|  | Invasive species are present. |  |  |  | Remove invasive species immediately and replace vegetation as needed. | Professional |  |
|  | Trees planted in the buffer and on wetland islands and peninsulas need watering during the first growing season |  |  |  | Consider watering every 3 days for first month, and then weekly during first year (April – October), depending on rainfall. | Owner or professional |  |
|  | Grass around the facility is overgrown |  |  |  | Mow (at least twice a year) to a height of 4”-9” high and remove grass clippings. | Owner or professional |  |
| Vegetation (continued) | Vegetation is dead or reinforcement planting is needed. |  |  |  | Remove and replace dead or dying vegetation. | Professional |  |
| Permanent Pool and Side Slopes | There is excessive trash and/or debris. |  |  |  | Remove immediately | Owner or professional |  |
|  | There is evidence of sparse vegetative cover, erosion or slumping side slopes. |  |  |  | Repair and stabilize physical damage, and reseed or plant additional vegetation. | Owner or professional |  |
|  | There is evidence of nuisance animals. |  |  |  | Animal burrows must be backfilled and compacted. Burrowing animals should be humanely removed frm area. | Owner or professional |  |
|  | There is significant sediment accumulation. |  |  |  | Conduct a bathymetric study to determine the impact to design volumes, and dredge | Professional |  |

| Element of BMP | Potential Problem | Problem? Y / N | Investigate? Y / N | Repaired? Y / N | How to Fix Problem | Who Will Address Problem | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Riser/Principle Spillway and Low-Flow Orifice(s) | There is adequate access to the riser for maintenance. |  |  |  | Establish adequate access | Professional and, perhaps, the locality |  |
|  | Pieces of the riser are deteriorating, misaligned, broken or missing. |  |  |  | Repair immediately. | Professional |  |
|  | Adjustable control valves are accessible and operational. |  |  |  | Repair, as needed. | Professional |  |
|  | Reverse-slope pipes and flashboard risers are in good condition. |  |  |  | Repair, as needed. | Professional |  |
|  | Seepage into conduit |  |  |  | Seal conduit | Professional |  |
|  | There is evidence of clogging |  |  |  | Clear blockages of the riser or orifice(s) and make other adjustments needed to meet the approved design specs. | Professional |  |
|  | There is excessive trash, debris, or other obstructions in the trash rack. |  |  |  | Remove immediately. | Owner or professional |  |
| Dam/ Embankment and Abutments | There is sparse veg. cover, settlement, cracking, bulging, misalignment, erosion rills deeper than 2 inches, or sloughing. |  |  |  | Repair and restabilize immediately, especially after major storms. | Professional |  |
|  | There are soft spots, seepage, boggy areas or sinkholes. |  |  |  | Reinforce, fill and stabilize immediately. |  |  |
|  | There is evidence of nuisance animals. |  |  |  | Animal burrows must be backfilled and compacted. Burrowing animals should be humanely removed from the area. |  |  |
|  | There is woody vegetation on the embankment. |  |  |  | Removal of woody species near or on the embankment and maintenance access areas should be done when discovered, but at least every 2 years. |  |  |
| Overflow/Emergency Spillway | There is woody growth on the spillway. |  |  |  | Removal of woody species near or on the emergency spillway should be done when discovered, but at least every 2 years. | Owner or professional |  |
|  | There is excessive trash, debris, or other obstructions. |  |  |  | Remove immediately. | Owner or professional |  |
|  | There is evidence of erosion/backcutting |  |  |  | Repair erosion damage and reseed | Owner or professional |  |
|  | There are soft spots, seepage or sinkholes. |  |  |  | Reinforce, fill and stabilize immediately. | Owner or professional |  |
|  | Only one layer of stone armoring exists above the native soil. |  |  |  | Reinforce rip-rap or other armoring materials. | Professional |  |

| Element of BMP | Potential Problem | Problem? Y / N | Investigate? Y / N | Repaired? Y / N | How to Fix Problem | Who Will Address Problem | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Outlet | The outlet provides a stable conveyance from the pond. |  |  |  | Stabilize immediately, as needed, and clear blockages. | Owner or professional |  |
|  | There is woody growth within 5 feet of the outlet pipe barrel. |  |  |  | Prune vegetation back to leave a clear discharge area. | Owner or Professional |  |
|  | There is excessive trash, debris, or other obstructions. |  |  |  | Remove immediately. | Owner or professional |  |
|  | There are excessive sediment deposits at the outlet. |  |  |  | Remove sediment. | Professional |  |
|  | Discharge is causing undercutting, erosion or displaced rip-rap at or around the outlet. |  |  |  | Repair, reinforce or replace rip rap as needed, and restabilize. | Professional |  |
| Overall | Access to the facility or its components is adequate. |  |  |  | Establish adequate access. Remove woody vegetation and debris that may block access. Ensure that hardware can be opened and operated. | Professional and, perhaps, the locality |  |
|  | Fences are inadequate |  |  |  | Collapsed fences must be restored to an upright position. Jagged edges and damaged fences must be repaired or replaced. | Professional |  |
|  | Water levels in one or more cells are abnormally high or low. |  |  |  | Clear blockages of the riser or orifice(s) and make other adjustments needed to meet the approved design specifications. | Professional |  |
|  | Complaints from local residents |  |  |  | Correct real problems. | Owner or professional |  |
|  | Mosquito proliferation |  |  |  | Eliminate stagnant pools and stock the basin with mosquito fish to provide natural mosquito & midge control. Treat for mosquitoes as needed. If spraying, then use mosquito larvicide, (e.g., Bacillus thurendensis or Altoside formulations) only if absolutely necessary. | Owner or professional |  |
| Overall (continued) | Encroachment on the pond or easement by buildings or other structures |  |  |  | Inform involved property owners of BMPs status ; clearly mark the boundaries of the receiving pervious area, as needed | Owner or professional (and perhaps the locality) |  |
|  | Safety signage is not adequate. |  |  |  | Provide sufficient, legible safety signage. | Owner or professional |  |